本科实验报告

| 课程名称： | 数字逻辑设计 |
| --- | --- |
| 姓 名： | 封和易 |
| 学 院： | 竺可桢学院 |
| 系： | 混合班 |
| 专 业： | 计算机科学与技术 |
| 学 号： | 3250104494 |
| 指导教师： | 马德 |

2026年 5 月 6 日

浙江大学实验报告

课程名称：           数字逻辑设计            实验类型： 模拟电路实验

实验项目名称：                  同步时序电路设计

学生姓名：   封和易    专业：  计算机科学与技术  学号：  3250104494

同组学生姓名：        宋雨恒、张旭          指导老师：     马德

实验地点：        紫金港东4-509          实验日期： 2026 年  5 月 6 日

实验目的：

- 掌握典型同步时序电路的工作原理和设计方法

- 掌握时序电路的激励函数、状态图、状态方程的运用

- 掌握用Verilog进行有限状态机的设计、调试、仿真

- 掌握用FPGA实现时序电路功能

子实验1：原理图方式设计4位同步二进制计数器

操作方法与实验步骤

实验数据记录和处理

Logisim实现电路图如下：

Main代码如下：

top.v

`timescale 1ns / 1ps

module top #(

parameter CNT_1S_MAX = 32'd49_999_999

)(

input  wire       clk,

output wire [3:0] AN,

output wire [7:0] SEGMENT,

output wire [0:0] LED

);

wire clk_1s;

wire QA;

wire QB;

wire QC;

wire QD;

wire RC;

wire [3:0] cnt4;

counter_1s #(

.CNT_MAX(CNT_1S_MAX)

) u_counter_1s (

.clk(clk),

.clk_1s(clk_1s)

);

Counter4b u_counter4b (

.QA(QA),

.QB(QB),

.QC(QC),

.QD(QD),

.RC(RC),

.clk(clk_1s)

);

assign cnt4 = {QA, QB, QC, QD};

DispNum u_disp (

.Hex(cnt4),

.AN(AN),

.SEGMENT(SEGMENT)

);

assign LED[0] = RC;

endmodule

counter_1s.v

`timescale 1ns / 1ps

module counter_1s #(

parameter CNT_MAX = 32'd49_999_999

)(

input  wire clk,

output reg  clk_1s

);

reg [31:0] cnt;

initial begin

cnt = 32'd0;

clk_1s = 1'b0;

end

always @(posedge clk) begin

if (cnt == CNT_MAX) begin

cnt <= 32'd0;

clk_1s <= ~clk_1s;

end

else begin

cnt <= cnt + 1'b1;

end

end

endmodule

DispNum.v

`timescale 1ns / 1ps

module DispNum(

input  wire [3:0] Hex,

output wire [3:0] AN,

output wire [7:0] SEGMENT

);

assign AN = 4'b1110;          // only the lowest digit is enabled

assign SEGMENT = seg7(Hex);   // SEGMENT[7:0] = {p,g,f,e,d,c,b,a}

function [7:0] seg7;

input [3:0] x;

begin

case (x)

4'h0: seg7 = 8'b11000000;

4'h1: seg7 = 8'b11111001;

4'h2: seg7 = 8'b10100100;

4'h3: seg7 = 8'b10110000;

4'h4: seg7 = 8'b10011001;

4'h5: seg7 = 8'b10010010;

4'h6: seg7 = 8'b10000010;

4'h7: seg7 = 8'b11111000;

4'h8: seg7 = 8'b10000000;

4'h9: seg7 = 8'b10010000;

4'hA: seg7 = 8'b10001000;

4'hB: seg7 = 8'b10000011;

4'hC: seg7 = 8'b11000110;

4'hD: seg7 = 8'b10100001;

4'hE: seg7 = 8'b10000110;

4'hF: seg7 = 8'b10001110;

default: seg7 = 8'b11111111;

endcase

end

endfunction

endmodule

Counter4b.v

/******************************************************************************

** Logisim-evolution goes FPGA automatic generated Verilog code             **

** https://github.com/logisim-evolution/                                    **

**                                                                          **

** Component : Counter4b                                                    **

**                                                                          **

*****************************************************************************/

module Counter4b( QA,

QB,

QC,

QD,

RC,

clk );

/*******************************************************************************

** The inputs are defined here                                                **

*******************************************************************************/

input clk;

/*******************************************************************************

** The outputs are defined here                                               **

*******************************************************************************/

output QA;

output QB;

output QC;

output QD;

output RC;

/*******************************************************************************

** The wires are defined here                                                 **

*******************************************************************************/

wire s_logisimNet0;

wire s_logisimNet1;

wire s_logisimNet10;

wire s_logisimNet11;

wire s_logisimNet12;

wire s_logisimNet13;

wire s_logisimNet14;

wire s_logisimNet15;

wire s_logisimNet16;

wire s_logisimNet17;

wire s_logisimNet18;

wire s_logisimNet19;

wire s_logisimNet2;

wire s_logisimNet3;

wire s_logisimNet4;

wire s_logisimNet5;

wire s_logisimNet6;

wire s_logisimNet7;

wire s_logisimNet8;

wire s_logisimNet9;

/*******************************************************************************

** The module functionality is described here                                 **

*******************************************************************************/

/*******************************************************************************

** Here all input connections are defined                                     **

*******************************************************************************/

assign s_logisimNet5 = clk;

/*******************************************************************************

** Here all output connections are defined                                    **

*******************************************************************************/

assign QA = s_logisimNet3;

assign QB = s_logisimNet1;

assign QC = s_logisimNet2;

assign QD = s_logisimNet4;

assign RC = s_logisimNet0;

/*******************************************************************************

** Here all in-lined components are defined                                   **

*******************************************************************************/

// Constant

assign  s_logisimNet13  =  1'b0;

// Constant

assign  s_logisimNet12  =  1'b0;

// Constant

assign  s_logisimNet15  =  1'b0;

// Constant

assign  s_logisimNet14  =  1'b0;

// Constant

assign  s_logisimNet17  =  1'b0;

// Constant

assign  s_logisimNet16  =  1'b0;

// Constant

assign  s_logisimNet19  =  1'b0;

// Constant

assign  s_logisimNet18  =  1'b0;

/*******************************************************************************

** Here all normal components are defined                                     **

*******************************************************************************/

AND_GATE_3_INPUTS #(.BubblesMask(3'b000))

GATES_1 (.input1(s_logisimNet1),

.input2(s_logisimNet2),

.input3(s_logisimNet4),

.result(s_logisimNet7));

AND_GATE #(.BubblesMask(2'b00))

GATES_2 (.input1(s_logisimNet2),

.input2(s_logisimNet4),

.result(s_logisimNet8));

AND_GATE_4_INPUTS #(.BubblesMask(4'h0))

GATES_3 (.input1(s_logisimNet3),

.input2(s_logisimNet1),

.input3(s_logisimNet2),

.input4(s_logisimNet4),

.result(s_logisimNet0));

XOR_GATE_ONEHOT #(.BubblesMask(2'b00))

GATES_4 (.input1(s_logisimNet3),

.input2(s_logisimNet7),

.result(s_logisimNet9));

XOR_GATE_ONEHOT #(.BubblesMask(2'b00))

GATES_5 (.input1(s_logisimNet1),

.input2(s_logisimNet8),

.result(s_logisimNet10));

XOR_GATE_ONEHOT #(.BubblesMask(2'b00))

GATES_6 (.input1(s_logisimNet2),

.input2(s_logisimNet4),

.result(s_logisimNet11));

D_FLIPFLOP #(.invertClockEnable(0))

MEMORY_7 (.clock(s_logisimNet5),

.d(s_logisimNet9),

.preset(s_logisimNet13),

.q(s_logisimNet3),

.qBar(),

.reset(s_logisimNet12),

.tick(1'b1));

D_FLIPFLOP #(.invertClockEnable(0))

MEMORY_8 (.clock(s_logisimNet5),

.d(s_logisimNet10),

.preset(s_logisimNet15),

.q(s_logisimNet1),

.qBar(),

.reset(s_logisimNet14),

.tick(1'b1));

D_FLIPFLOP #(.invertClockEnable(0))

MEMORY_9 (.clock(s_logisimNet5),

.d(s_logisimNet11),

.preset(s_logisimNet17),

.q(s_logisimNet2),

.qBar(),

.reset(s_logisimNet16),

.tick(1'b1));

D_FLIPFLOP #(.invertClockEnable(0))

MEMORY_10 (.clock(s_logisimNet5),

.d(s_logisimNet6),

.preset(s_logisimNet19),

.q(s_logisimNet4),

.qBar(s_logisimNet6),

.reset(s_logisimNet18),

.tick(1'b1));

endmodule

Simulation Sources：

Counter4b_Test.v

`timescale 1ns / 1ps

module Counter4b_Test;

reg clk;

wire QA, QB, QC, QD, RC;

reg [3:0] exp;

integer i;

Counter4b uut (

.QA(QA),

.QB(QB),

.QC(QC),

.QD(QD),

.RC(RC),

.clk(clk)

);

initial begin

clk = 0;

forever #5 clk = ~clk;      // 10ns period

end

initial begin

exp = 4'b0000;

#2;

$display("time\tclk\tQA QB QC QD\tRC");

for (i = 0; i < 20; i = i + 1) begin

@(posedge clk);

#1;

$display("%0t\t%b\t%b  %b  %b  %b\t%b",

$time, clk, QA, QB, QC, QD, RC);

if ({QA, QB, QC, QD} !== exp) begin

$display("ERROR: expected %b, got %b",

exp, {QA, QB, QC, QD});

end

if (RC !== (&exp)) begin

$display("ERROR: RC expected %b, got %b",

(&exp), RC);

end

exp = exp + 1'b1;

end

$finish;

end

endmodule

top_Test.v:

`timescale 1ns / 1ps

module Counter4b_Test;

reg clk;

wire QA, QB, QC, QD, RC;

reg [3:0] exp;

integer i;

Counter4b uut (

.QA(QA),

.QB(QB),

.QC(QC),

.QD(QD),

.RC(RC),

.clk(clk)

);

initial begin

clk = 0;

forever #5 clk = ~clk;      // 10ns period

end

initial begin

exp = 4'b0000;

#2;

$display("time\tclk\tQA QB QC QD\tRC");

for (i = 0; i < 20; i = i + 1) begin

@(posedge clk);

#1;

$display("%0t\t%b\t%b  %b  %b  %b\t%b",

$time, clk, QA, QB, QC, QD, RC);

if ({QA, QB, QC, QD} !== exp) begin

$display("ERROR: expected %b, got %b",

exp, {QA, QB, QC, QD});

end

if (RC !== (&exp)) begin

$display("ERROR: RC expected %b, got %b",

(&exp), RC);

end

exp = exp + 1'b1;

end

$finish;

end

endmodule

Running Simulation结果如下：

实验结果与分析

- 在 Logisim/Digital 中完成 4 位同步二进制计数器原理图设计，并导出 Counter4b.v，再在 Vivado 中调用该模块完成顶层设计。

- 仿真结果显示，{QA,QB,QC,QD} 按如下顺序变化：

0000 → 0001 → 0010 → ... → 1111 → 0000

说明 4 位同步二进制计数功能正确。

- RC 仅在：

QA QB QC QD = 1111

时为 1，其余状态为 0，说明进位输出逻辑正确。

- counter_1s.v 将 100 MHz 时钟分频为 1 Hz，使计数器每秒自增一次。DispNum.v 将计数值显示在最低位数码管上，AN = 1110，表示只选通最低位。

- 下载到 FPGA 后，数码管能够循环显示：

0 → 1 → 2 → ... → F → 0

且 LED[0] 在计数值为 F 时点亮。仿真结果与硬件现象一致，说明实验 1 设计正确。

子实验2：以行为描述方式设计16位可逆二进制同步计数器

操作方法与实验步骤

实验数据记录和处理

Main代码如下：

Top.v:

`timescale 1ns / 1ps

module top #(

parameter CLK100MS_MAX = 32'd4_999_999

)(

input  wire       clk,

input  wire [0:0] SW,

output wire [3:0] AN,

output wire [7:0] SEGMENT,

output wire [0:0] LED

);

wire clk_cnt;

wire [15:0] cnt;

wire Rc;

clk_100ms #(

.CNT_MAX(CLK100MS_MAX)

) u_clk_100ms (

.clk(clk),

.clk_out(clk_cnt)

);

RevCounter u_counter (

.clk(clk_cnt),

.s(SW[0]),

.cnt(cnt),

.Rc(Rc)

);

DispNum u_disp (

.clk(clk),

.Hexs(cnt),

.AN(AN),

.SEGMENT(SEGMENT)

);

assign LED[0] = Rc;

endmodule

RevCounter.v:

`timescale 1ns / 1ps

module RevCounter(

input  wire        clk,

input  wire        s,

output reg  [15:0] cnt,

output wire        Rc

);

initial begin

cnt = 16'h0000;

end

assign Rc = (s & (&cnt)) | (~s & (~|cnt));

always @(posedge clk) begin

if (s)

cnt <= cnt + 1'b1;

else

cnt <= cnt - 1'b1;

end

endmodule

clk_100ms.v:

`timescale 1ns / 1ps

module clk_100ms #(

parameter CNT_MAX = 32'd4_999_999

)(

input  wire clk,

output reg  clk_out

);

reg [31:0] cnt;

initial begin

cnt = 32'd0;

clk_out = 1'b0;

end

always @(posedge clk) begin

if (cnt == CNT_MAX) begin

cnt <= 32'd0;

clk_out <= ~clk_out;

end

else begin

cnt <= cnt + 1'b1;

end

end

endmodule

DispNum.v：

`timescale 1ns / 1ps

module DispNum(

input  wire        clk,

input  wire [15:0] Hexs,

output reg  [3:0]  AN,

output reg  [7:0]  SEGMENT

);

reg [17:0] scan_cnt;

wire [1:0] scan_sel;

reg [3:0] hex_now;

initial begin

scan_cnt = 18'd0;

AN = 4'b1110;

SEGMENT = 8'b11111111;

hex_now = 4'h0;

end

always @(posedge clk) begin

scan_cnt <= scan_cnt + 1'b1;

end

assign scan_sel = scan_cnt[17:16];

always @(*) begin

case (scan_sel)

2'b00: begin

AN = 4'b1110;

hex_now = Hexs[3:0];

end

2'b01: begin

AN = 4'b1101;

hex_now = Hexs[7:4];

end

2'b10: begin

AN = 4'b1011;

hex_now = Hexs[11:8];

end

2'b11: begin

AN = 4'b0111;

hex_now = Hexs[15:12];

end

default: begin

AN = 4'b1110;

hex_now = 4'h0;

end

endcase

end

always @(*) begin

case (hex_now)

4'h0: SEGMENT = 8'b11000000;

4'h1: SEGMENT = 8'b11111001;

4'h2: SEGMENT = 8'b10100100;

4'h3: SEGMENT = 8'b10110000;

4'h4: SEGMENT = 8'b10011001;

4'h5: SEGMENT = 8'b10010010;

4'h6: SEGMENT = 8'b10000010;

4'h7: SEGMENT = 8'b11111000;

4'h8: SEGMENT = 8'b10000000;

4'h9: SEGMENT = 8'b10010000;

4'hA: SEGMENT = 8'b10001000;

4'hB: SEGMENT = 8'b10000011;

4'hC: SEGMENT = 8'b11000110;

4'hD: SEGMENT = 8'b10100001;

4'hE: SEGMENT = 8'b10000110;

4'hF: SEGMENT = 8'b10001110;

default: SEGMENT = 8'b11111111;

endcase

end

endmodule

Simulation Sources：

`timescale 1ns / 1ps

module top_Test;

reg clk;

reg [0:0] SW;

wire [3:0] AN;

wire [7:0] SEGMENT;

wire [0:0] LED;

wire sim_clk_cnt;

wire [15:0] sim_cnt;

wire sim_Rc;

integer i;

top #(

.CLK100MS_MAX(32'd4)

) uut (

.clk(clk),

.SW(SW),

.AN(AN),

.SEGMENT(SEGMENT),

.LED(LED)

);

assign sim_clk_cnt = uut.clk_cnt;

assign sim_cnt = uut.cnt;

assign sim_Rc = uut.Rc;

initial begin

clk = 1'b0;

forever #5 clk = ~clk;

end

task show_state;

input [127:0] tag;

begin

$display("%0t  %s  SW0=%b  cnt=%h  Rc=%b  LED0=%b  AN=%b  SEGMENT=%b",

$time, tag, SW[0], sim_cnt, sim_Rc, LED[0], AN, SEGMENT);

end

endtask

initial begin

SW[0] = 1'b1;

$display("==== Normal forward counting ====");

for (i = 0; i < 6; i = i + 1) begin

@(posedge sim_clk_cnt);

#1;

show_state("UP_NORMAL");

end

SW[0] = 1'b0;

$display("==== Normal reverse counting ====");

for (i = 0; i < 6; i = i + 1) begin

@(posedge sim_clk_cnt);

#1;

show_state("DOWN_NORMAL");

end

$display("==== Forward carry Rc test ====");

@(negedge sim_clk_cnt);

SW[0] = 1'b1;

force uut.u_counter.cnt = 16'hfffe;

#1;

release uut.u_counter.cnt;

@(posedge sim_clk_cnt);

#1;

show_state("UP_TO_FFFF");

@(posedge sim_clk_cnt);

#1;

show_state("UP_TO_0000");

$display("==== Reverse borrow Rc test ====");

@(negedge sim_clk_cnt);

SW[0] = 1'b0;

force uut.u_counter.cnt = 16'h0001;

#1;

release uut.u_counter.cnt;

@(posedge sim_clk_cnt);

#1;

show_state("DOWN_TO_0000");

@(posedge sim_clk_cnt);

#1;

show_state("DOWN_TO_FFFF");

$finish;

end

endmodule

Constraints：

## ============================================================

## Clock: 100MHz

## ============================================================

create_clock -name clk100MHZ -period 10.0 [get_ports clk]

set_property PACKAGE_PIN AC18 [get_ports clk]

set_property IOSTANDARD LVCMOS18 [get_ports clk]

## ============================================================

## Switch: SW[0]

## SW[0] = 1: count up

## SW[0] = 0: count down

## ============================================================

set_property PACKAGE_PIN AA10 [get_ports {SW[0]}]

set_property IOSTANDARD LVCMOS15 [get_ports {SW[0]}]

## ============================================================

## Seven-segment digit enable: AN[3:0]

## Active low

## ============================================================

set_property PACKAGE_PIN AD21 [get_ports {AN[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {AN[0]}]

set_property PACKAGE_PIN AC21 [get_ports {AN[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {AN[1]}]

set_property PACKAGE_PIN AB21 [get_ports {AN[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {AN[2]}]

set_property PACKAGE_PIN AC22 [get_ports {AN[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {AN[3]}]

## ============================================================

## Seven-segment segment output

## SEGMENT[0] = a

## SEGMENT[1] = b

## SEGMENT[2] = c

## SEGMENT[3] = d

## SEGMENT[4] = e

## SEGMENT[5] = f

## SEGMENT[6] = g

## SEGMENT[7] = p

## Active low

## ============================================================

set_property PACKAGE_PIN AB22 [get_ports {SEGMENT[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[0]}]

set_property PACKAGE_PIN AD24 [get_ports {SEGMENT[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[1]}]

set_property PACKAGE_PIN AD23 [get_ports {SEGMENT[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[2]}]

set_property PACKAGE_PIN Y21 [get_ports {SEGMENT[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[3]}]

set_property PACKAGE_PIN W20 [get_ports {SEGMENT[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[4]}]

set_property PACKAGE_PIN AC24 [get_ports {SEGMENT[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[5]}]

set_property PACKAGE_PIN AC23 [get_ports {SEGMENT[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[6]}]

set_property PACKAGE_PIN AA22 [get_ports {SEGMENT[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {SEGMENT[7]}]

set_property PACKAGE_PIN W23 [get_ports {LED[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {LED[0]}]

Running Simulation结果如下：

Generate Bitstream后，在Hardware上的表现符合预期：

实验结果与分析

- 使用 Verilog 行为描述方式完成 RevCounter.v。SW[0] 控制计数方向：

SW[0] = 1：正向计数
SW[0] = 0：反向计数

- 仿真结果显示，正向计数时：

0000 → 0001 → 0002 → ...

反向计数时：

... → 0002 → 0001 → 0000 → FFFF

说明可逆计数功能正确。

- Rc 的仿真结果满足：

SW[0] = 1 且 cnt = FFFF 时，Rc = 1
SW[0] = 0 且 cnt = 0000 时，Rc = 1

说明正向进位和反向借位判断正确。

- clk_100ms.v 将 100 MHz 时钟分频，使计数器每 0.1 s 改变一次。DispNum.v 将 16 位计数值分为 4 个十六进制位，动态显示在四位数码管上。

- 数码管扫描位选为：

1110 → 1101 → 1011 → 0111

段选采用低电平有效，且 SEGMENT[0]~SEGMENT[7] 分别对应 a,b,c,d,e,f,g,p，与前序实验约束一致。

- 下载到 FPGA 后，拨动 SW[0] 可以切换加计数和减计数；LED[0] 与 Rc 一致，在进位或借位状态点亮。仿真结果与硬件现象一致，说明实验 2 设计正确。